ENGINEERING CHANGE NOTICE   DRAFT — FOR REVIEW

TUUCI — Meritage product family · Prepared from the Meritage 3-Seater Engineering Improvement Plan (06/18/2026)

| ECN No. | ECN-2026-____ (assigned on release) | Date | 07/13/2026 |
| --- | --- | --- | --- |
| Originator | Douglas O'Keefe — Continuous Improvement | Change class | Major — BOM, drawings, supplier process |
| Product(s) | Meritage 3-Seater Sofa (primary). Connector system is shared across Meritage lines — all Meritage SKUs using flat horizontal connectors are affected. |  |  |
| Priority | High — recovers ~24 min labor/unit; two items relieve the seat-frame bottleneck | Requested effectivity | First supplier PO after approval (date: ____) |

1  REASON FOR CHANGE

Meritage flat connectors arrive from the Vietnam supplier as one generic configuration and are hand-reworked on the line before use (screw swaps, orientation flips, hole drilling), and the high-volume leg piece is assembled in-house. This adds ~24 minutes of non-value-added labor per unit, part of it directly on the seat-frame bottleneck, so every minute recovered there raises line throughput — not just labor. Root cause is a BOM/documentation gap: three real connector configurations exist but the BOM lists only two, and the connector CAD drawings carry an incorrect part number, so the supplier cannot ship parts pre-configured.

2  CURRENT CONDITION → REQUESTED CHANGE

Item A — Correct the connector BOM (3 configurations, unique part numbers) — prerequisite for B–D

| Current condition | Requested change |
| --- | --- |
| BOM lists 2 connector types; all arrive as one generic design. Connector CAD drawings show P/N 1012553, which matches neither the BOM nor the floor. Two configurations share a part number. | BOM and connector CAD drawings list 3 unique configurations with their own P/Ns: • Front (same both sides): 1015301 — qty 2 • Back-left: 1015302 — qty 1 • Back-right: 1016592 — qty 1 Update connector drawings to the correct P/Ns and configurations (left/right defined). |

Reference: the seat-frame assembly drawing already shows all three P/Ns correctly (1015301 ×2 front, 1015302 + 1016592 back); drawing TU-530 Rev A likewise BOMs the three connector assemblies separately (Config 1/2/3). Only the BOM and the connector-level drawings lag.

Item B — Eliminate screw swaps & reorientation (ship pre-configured)

| Current condition | Requested change |
| --- | --- |
| Type 1 (front) arrives with screw 40200; operator removes it and installs 1012919 on every connector. Type 2 (back) arrives with 1012934; operator swaps to 40204. Half of Type 2 also need the screw-entry side flipped by hand. Rework ≈ 11 min 40 s per unit, zero value added. | Each configuration ships from the supplier already fitted with the correct screw and orientation: • 1015301 with 1012919 • 1015302 / 1016592 with 40204, left/right orientation per drawing Screw specs to be validated by Engineering before release (see §6). |

Item C — Pre-drill connector holes for leg insertion (bottleneck relief)

| Current condition | Requested change |
| --- | --- |
| Operators manually drill 4 holes into the flat connectors for leg-piece insertion during seat-frame assembly — the line's top bottleneck. ≥ 5 min per frame. | Connectors arrive with leg-insertion holes pre-drilled per drawing (hole size/position to be added to the updated connector drawings in Item A). |

Item D — Receive the high-volume leg piece pre-assembled

| Current condition | Requested change |
| --- | --- |
| Connector operator assembles the leg piece in-house, ~8 min per unit. Two variants exist; the basic (non-wheeled) variant is ~90% of volume. | The basic leg piece arrives pre-assembled from the supplier under its own P/N (assign: ____). The wheeled variant (~10%) remains assembled in-house. Gate: supplier finish must meet TUUCI paint spec (see §6). |

3  AFFECTED ITEMS

| Item | Type | Change |
| --- | --- | --- |
| Meritage 3-Seater BOM (and shared-connector Meritage BOMs) | BOM | 2 connector types → 3 unique P/Ns (1015301 / 1015302 / 1016592) with quantities 2/1/1; add pre-assembled leg P/N |
| Connector CAD drawing(s) currently P/N 1012553 | Drawing | Correct P/Ns; define left/right configurations; add fitted-screw callouts; add pre-drilled leg-insertion holes (size, position, qty 4) |
| Screws 40200, 1012934 | Component | Removed from connector configurations (superseded by 1012919 / 40204, pending validation) |
| Basic leg piece | Part / sourcing | New pre-assembled P/N; supplier assembly + paint-spec qualification |
| Supplier POs / work instructions (Vietnam) | Procurement | Order by configuration P/N; pre-fit screws; pre-drill holes; leg pre-assembly |
| Line SWI — connector prep & seat-frame stations | Standard work | Remove screw-swap / reorientation / drilling / leg-assembly steps after cut-in; re-balance line with recovered labor |

4  LABOR IMPACT (MEASURED / ESTIMATED)

| Item | Recovery per unit | Where it lands |
| --- | --- | --- |
| B — screw swaps + reorientation | ~11 min 40 s | Connector operator (redeploy to bottleneck) |
| C — pre-drilled holes | ≥ 5 min | Seat-frame bottleneck — direct throughput gain |
| D — pre-assembled leg (90% of units) | ~8 min (≈7.2 blended) | Connector operator + manufacturing/paint upstream |
| Total | ≈ 24 min/unit | Concentrated on / redeployable to the bottleneck |

5  DISPOSITION OF EXISTING MATERIAL

| Material | Disposition |
| --- | --- |
| Generic connectors in house / in transit | Use as is — continue current line rework until pre-configured stock cuts in (no scrap) |
| Screw stock 40200 / 1012934 | Use for service/other products if applicable; excess to be dispositioned by Supply Chain |
| Leg-piece components in house | Consume through cut-in of pre-assembled part |

6  OPEN ITEMS / RISKS (TO CLOSE BEFORE RELEASE)

Screw spec validation: 1012919 and 40204 are what operators use today and are confirmed to work — a working baseline, not a validated spec. Engineering to confirm optimal spec before formalizing in the BOM.

Paint spec (Item D): confirm the supplier can deliver the basic leg piece finished to TUUCI paint spec through the Vietnam supply chain. This is the gate on Item D.

Hole spec (Item C): engineering to formalize drill size and positions on the updated connector drawings.

Effectivity plan: define cut-in (PO number / lot) and dual-stock handling during transition.

7  IMPLEMENTATION SEQUENCE

Approve BOM correction (Item A) — enables everything downstream

Release updated connector drawings (P/Ns, screws, orientation, holes)

Supplier changeover: pre-configured connectors + pre-drilled holes (Items B/C); qualify leg paint and cut in Item D

Re-balance the cell to takt with the recovered labor; update SWIs

8  APPROVALS

| Function | Name | Signature | Date |
| --- | --- | --- | --- |
| Originator — Continuous Improvement | Douglas O'Keefe |  |  |
| Engineering |  |  |  |
| Supply Chain / Purchasing |  |  |  |
| Quality |  |  |  |
| Production |  |  |  |

References: Meritage 3-Seater Engineering Improvement Plan (O'Keefe, 06/18/2026) · Seat-frame assembly drawing (shows 1015301 ×2 / 1015302 / 1016592) · TU-530 Rev A 'Seat, MER, Ottoman No Arms Subassy' (BOMs Config 1/2/3 connector assemblies separately) · Connector CAD drawings currently marked 1012553.